TECHNICAL STUDY GUIDE

LiuQP

Mathematical Derivation and Theoretical Guide

A section-by-section explanation of Liu and Yim (2021)

Scope  This guide covers the theoretical content through Algorithm 1, including equations (1)-(27), the sphere-tree obstacle model, closed-form separating planes, redundant-sphere removal, the integrated quadratic program, and the near/contact objective-constraint switch.

Source paper

Chao Liu and Mark Yim, A Quadratic Programming Approach to Manipulation in Real-Time Using Modular Robots

Prepared as an English-language study document | August 2026

How to Read This Guide

The paper does not merely place collision constraints in a quadratic program. Its real-time method is a chain of four coupled ideas:

- Build the kinematics of an arbitrary tree-structured modular robot from module and connection graphs.

- Convert task tracking into a joint-velocity equality or a soft least-squares objective.

- Approximate robot modules and obstacles with spheres, construct local separating planes in closed form, and remove obstacle spheres made redundant by those planes.

- Solve a convex velocity-level QP, execute one short step, and rebuild all state-dependent quantities at the next control cycle.

Central correction  The separating-plane reduction is not an optional implementation optimization. It is part of the paper's theoretical collision-avoidance construction and appears explicitly in Section 4.2, equation (24), and line 8 of Algorithm 1.

Document Map

- Problem formulation and contribution

- Graph-based kinematics and equations (1)-(13)

- Feedback control and equations (14)-(20)

- Workspace boundaries and equations (21)-(22)

- Sphere-tree obstacles, separating planes, and equation (23)

- Redundant-sphere removal and the active set S_i

- Integrated and sequential QPs, equations (24)-(27)

- Algorithm 1, assumptions, limitations, and reproduction requirements

Notation

| Symbol | Meaning |
| --- | --- |
| W | World frame |
| Mi | Body frame of robot module i |
| C | Connector frame |
| Fi | Task frame or end-effector frame i |
| gAB∈SE3 | Homogeneous transformation describing frame B relative to frame A |
| Θ, Θ | Joint position vector and joint velocity decision vector |
| xi, xi | Twist coordinates and their SE3 matrix representation |
| Vs, Js | Spatial twist and spatial Jacobian |
| pMi, ri | Center and radius of the conservative sphere for module i |
| oj,rjo | Center and radius of obstacle sphere j |
| S, Si | All obstacle spheres and the active subset retained for module i |
| K≻0 | Positive-definite task-error feedback gain |
| λ, μij | Task-tracking and near-obstacle penalty weights |
| dmin, γij≤0 | Near-obstacle threshold and contact repulsion threshold |

Problem Formulation and Contribution

Conventional motion planning is commonly divided into a path planner, a trajectory generator, and a tracking controller. This separation is effective for many robots, but it rarely makes all three components real-time in a high-dimensional modular system.

A modular robot may contain many modules, each with one or more degrees of freedom. The number of possible connection configurations grows rapidly, and a single configuration may contain multiple kinematic chains whose motions share joints and are therefore coupled.

Liu and Yim replace full-path planning with a repeated velocity-level optimization. At each short control interval, the method computes a joint-velocity vector, executes it, observes the new state, and reconstructs the local convex problem.

Theoretical contribution: A universal graph-derived kinematic model, a feedback-control task formulation, linear workspace and obstacle constraints, dynamic redundant-sphere removal, and a sequentially updated convex QP.

Planner type: A local online planner/controller. It may also be integrated offline by one-step Euler integration, but it is not a globally complete path planner.

Graph-Based Kinematics for Modular Robots

Configuration graph and module graph

The undirected configuration graph G=V,E describes connectivity: every vertex is a module, and every edge is a physical connection between two modules. It does not yet provide the transformations needed for forward kinematics.

Each module type therefore has a directed module graph Gm=Vm,Em. Its vertices are rigid bodies, including the module body and connector bodies. Its directed edges store fixed or joint-dependent homogeneous transformations. Edge direction indicates the forward-kinematics direction.

Starting from a base module fixed to the world frame, a breadth-first traversal combines module graphs and fixed connector-to-connector transformations into the full kinematics graph GK=VK,EK. A path from W to a task frame F defines a kinematic chain GK:W⇝F.

Topology assumption: The paper treats tree topologies without kinematic loops. A closed chain would require additional closure constraints that are not derived in this work.

Module kinematics: product of exponentials

For a module body frame M and connector frame C, every joint coordinate θi has a twist xi_theta_i expressed relative to M. The module-to-connector transform is written as a product of exponentials:

gMC=gMCΘC=∏iexpξθiθiCgMC0	(1)

Here gMC0 is the fixed zero-configuration geometry. The exponential factors are ordered along the local chain from M to C. If no joint lies between the two frames, gMC is constant, and g_CM = g_MC^(-1).

Conceptual guide: twists and the product of exponentials

A joint twist is best understood as a unit motion specification. For joint i, the six-dimensional twist is

ξi=viT,ωiTT∈R6

The angular component ωi specifies the rotation-axis direction. The linear component vi describes translation and, for a revolute joint, also encodes where the rotation axis lies. It is not necessarily the current linear velocity of the joint. The actual instantaneous twist is

Vi=ξiθi

Thus ξi specifies the direction or type of motion, θi specifies how fast the joint moves, and θi specifies how far it has moved.

Revolute and prismatic examples

For a revolute joint about the z-axis through the coordinate origin,

ω=0,0,1T, v=0,0,0T, ξ=0,0,0,0,0,1T

The twist describes pure rotation about z. For a prismatic joint translating along the x-axis,

v=1,0,0T, ω=0,0,0T, ξ=1,0,0,0,0,0T

The same six-dimensional representation therefore covers both rotational and translational joints.

Why the linear component can be nonzero for a revolute joint

Let the rotation axis have unit direction omega and pass through a point q. The linear component is

v=-ω×q

For a z-axis passing through q=100,

q=100, ω=001, v=-ω×q=-001×100=0-10

ξ=0-10001

This is still a purely revolute joint. The nonzero vrecords that the rotation axis does not pass through the chosen coordinate origin; points at the origin therefore move around that axis.

Hat map

The hat operator rewrites the six-vector ξ=vω as a 4-by-4 matrix so it can participate in homogeneous-transform calculations:

ξ=ω×v01×30

ω×=0-ωzωyωz0-ωx-ωyωx0

The skew-symmetric matrix implements the cross product: (Properties of the Cross Product)

ω×x=ω×x

For ω=001, the skew matrix and twist matrix are

ω×=0-10100000

ξ=0-100100000000000

What does the matrix exponential mean?

The matrix exponentially converts a joint's unit motion and displacement into a finite rigid-body transform:

expξθ=Rθpθ01∈SE3

Where Rθ :  Rotation produced by joint movement;

pθ: Translation resulting from joint movement.

More specifically:

expω×v00θ=expω×θ0θexpω×sv ds01

Where:

Rθ=expω×θ

pθ=0θexpω×sv ds

For a revolute joint about the z-axis through the origin,

expξθ=cosθ-sinθ00sinθcosθ0000100001

For a prismatic joint translating a distance theta along x, ξ=100000

if the distance travelled is θ, then:

expξθ=100θ010000100001

The same exponential notation therefore produces either a rotation, a translation, or a screw motion, depending on the twist.

The home transform

The matrix gMC0) is the connector pose Crelative to the module body Mwhen all relevant joint coordinates are zero. It records the module's fixed geometry. For example, a connector located 0.5 m along the module x-axis at zero configuration has

gMC0=1000.5010000100001

For a single joint, its current connector pose is

gMCθ=expξθgMC0

Worked one-joint example

Suppose the connector is initially at 1,0,0Tand the joint rotates about the world z-axis. Then

gMC0=1001010000100001

At theta = 90 degrees,

expξθ=0-100100000100001

gMC90∘=0-100100100100001The connector position has moved from 1,0,0Tto 0,1,0T.

Multiple joints and ordering

If the path from M to C crosses n joints, every joint contributes a finite transform:

gMCΘ=expξ1θ1expξ2θ2⋯expξnθngMC0
Matrix multiplication acts from right to left: the home pose is acted on first, followed by the joint transforms according to the chain convention. The order cannot be exchanged because three-dimensional rigid motions generally do not commute:

expξ1θ1expξ2θ2≠expξ2θ2expξ1θ1

The superscript C in θC denotes the subset of joints on the path from the module body M to connector C. It is a path label, not an exponent. For example, if only joints 1 and 2 affect connectors C1,

ΘC1=θ1θ2

In plain language, equation (1) says: start from the connector's zero-configuration geometry and apply each joint's rigid motion in chain order to obtain the connector's current position and orientation.

Chain position and spatial velocity

Equation (2): extracting the task-frame origin

Write the world-to-task homogeneous transform as

gWF=RWFpWF01

RWF is the orientation of task frame F relative to the world frame W, and pWF is the world-coordinate position of the origin of F. The origin of F has homogeneous coordinates 0,0,0,1T in its own frame. Transforming that point into W gives

pFW=gWF0001	(2)

The multiplication simply selects the last column of gWF. Therefore equation (2) extracts the task-frame origin from the complete pose matrix.

The transform gWF itself is obtained by multiplying every transform along the graph path from  W to F. For example,

gWF=gWM1gM1C1gC1M2gM2FEach factor changes coordinates between adjacent frames; their ordered product changes coordinates directly from F to  W.

Time differentiation and the multivariable chain rule

Every joint coordinate θij varies with time,

θij=θijt

so the end-frame transform is a multivariable function gWF=gWFθ11,θ12,…. Its derivative is

gWF=ddtgWFΘt

gWF=ij∂gWF∂θijθij

Here i identifies a module and j identifies a joint inside that module. The partial derivative states how the end-frame transform would change if only θij changed; multiplication by θijscales that unit effect by the joint's actual speed. This is the contribution of joint i,j to the rate of change in the end-effector pose. Summing all terms gives the total pose rate.

Why g can’t be converted into a twist directly?

The derivative g describes the rate of change of a matrix, but it is not yet in the standard se(3) twist form.

g=Rp01

Then, g=Rp00. The issues are:

- R itself is not a standard antisymmetric angular velocity matrix;

- The expression for g depends on the current pose g;

- We want to convert pose changes into a unified 6-dimensional velocity representation.

Therefore, the paper defines the spatial twist by

VWFs=gWFgWF-1

Right multiplication by gWF-1 maps the velocity at the current pose back to the identity, producing a standard rigid-body velocity expressed in world coordinates.

gg-1=Velocity of a rigid body expressed in the world coordinate system

The right multiplication by g-1can be understood as follows:

It maps the velocity at the current pose gback to the unit pose, thereby yielding the standard rigid-body velocity matrix.

More mathematically, g lies in the tangent space of SE3 at the current point g; after multiplication by g-1, it is transformed to the point of the unit element in the Lie algebra se3

If

g=R,p;0,1, g=R,p;0,0

Then

(xW=RxM+pxM=R-1xW-pR-1=RT,xM=RTxW-p=RTxW-RTp.)

xM1=RT-RTp0T1xW1
g-1=RT-RTp01

gg-1=RRT,p-RRTp;0,0=ωs×,vs;0,0

Proof: A rotation matrix satisfies:

RRT=I.

Take the derivative with respect to time on both sides:

ddtRRT=0.

RRT+RRT=0.

Note

RRT=RRTT.

Thus

RRT+RRTT=0.

i.e.

RRTT=-RRT

Therefore, RRTmust be a 3×3 antisymmetric matrix. Any three-dimensional antisymmetric matrix can be uniquely written as

ω×=0-ωzωyωz0-ωx-ωyωx0.

Therefore, there must be a vector ωs such that

RRT=ωs×

Because RRT is skew-symmetric, it represents the angular velocity ωs. The spatial twist's linear component is

vs=p - ωs×p

Consequently, vs is not always the velocity of the moving frame's origin. The actual velocity of a point at p is

p=vs+ωs×p

This is why equation (9) later applies the twist matrix to the homogeneous point coordinate.

Spatial versus body twists

Vs=gg-1, Vb=g-1g

The first expression is a spatial twist expressed in the world frame; the second is a body twist expressed in the moving body's own frame. The paper uses spatial twists because module positions, obstacle directions, and collision constraints are evaluated in world coordinates.

Deriving equation (3)

Substitute the chain-rule expression for g-dot into the definition of the spatial twist:

VWFs=∑i=1N∑j=1Ni∂gWF∂θijgWF-1θij	(3)

i.e.,

Vs=gg-1=i,j∂g∂θijg-1θij.

For the current configuration, each matrix inside parentheses is the unit spatial twist contributed by joint i,j, expressed in world coordinates:

ξ'ij=∂gWF∂θijgWF-1

VWFs=ijξ'ijθij

Thus, the total end-frame velocity is the sum of all joint-velocity contributions.

From the sum to the spatial Jacobian

Stacking the unit twists as columns and the joint rates as a vector converts the sum into a matrix product:

VWFs=JWFsΘWF	(4)

ΘWF=θ11⋯θ1N1θ21⋯θNNNT	(5)

JWFs=J1 J2 ⋯ JN=ξ'1ξ'2⋯ξ'n	(6)

Ji=∂g∂θi1g-1∨ ⋯ ∂g∂θiNig-1∨	(7)

⋅∨ is not a union operator but is called the vee operator. Its function is to convert a 4×4 twist matrix back into a 6×1 twist vector.

Proof:

Every single one here

∂g∂θijg-1

They are all 4×4 twist matrices of the form

ξ'ij=ω'ij×v'ij0T0.

However, we hope to ultimately use a standard six-dimensional velocity vector.

Vs=vsωs∈R6.

So, apply the vee operator to both sides:

Vs=i,j∂g∂θijg-1∨θij.

For module i, arrange the six-dimensional vectors corresponding to each joint in columnar form:

Ji=∂g∂θi1g-1∨⋯∂g∂θiNig-1∨.

This is the Jacobian matrix of module i.

If module i has Ni joints, then

Ji∈R6×Ni.

Each column represents the spatial velocity of the task coordinate system relative to the world coordinate system when this joint alone moves at 1 rad/s and all other joints remain stationary.

Thus, the contribution to the velocity of module i is

Vis=JiΘi,

In which,

Θi=θi1⋮θiNi.

Equivalently, the adjoint transformation maps each module-frame joint twist into the world frame, giving

ξ'ij=AdgWMiξij

Ji=ξ'i1 ξ'i2 ⋯ ξ'iNi	(8)

Calculating each column directly:

∂g∂θijg-1∨

This is quite cumbersome because it requires:

1.    Computing the partial derivatives of the transformation g for the entire kinematic chain;

2.    Calculating g-1;

3.    Perform matrix multiplication;

4.    Then apply to the vee operator.

However, the joint twist ξij is usually already known in the module coordinate system. All we really need to do is transform it from the module coordinate system to the world coordinate system.

This is the adjoint transformation:

ξ'ij=AdgWMiξij.

Where:

•    ξij: The twist of the joint in the module coordinate system Mi;

•    gWMi: The pose of the module coordinate system relative to the world coordinate system;

•    ξ'ij: The twist of the same joint motion expressed in the world coordinate system.

It satisfies:

∂g∂θijg-1∨=AdgWMiξij

Therefore, the Jacobian matrix can be written directly as:

Ji=ξ'i1⋯ξ'iNi.

It is no longer necessary to compute the partial derivatives column by column for the entire g.

Supplement of Proof:

First, let’s recall the definition of matrix transposition:

ω×x=ω×x.

In other words, ω×itself represents the linear operation “transpose with ω.”

I. Lemma 1:

Rω×RT=Rω×

The most fundamental property is: For a rotation matrix R∈SO3,

Ra×b=Ra×Rb

This means:

Performing the cross product followed by a rotation is equivalent to first rotating both vectors and then performing the cross product.

This is because rotation does not change the length, angle, or right-hand orientation of the vectors.

To prove that the matrices are equal, let’s take an arbitrary vector x and examine the action of the left-hand side on x:

Rω×RTx.

Step 1: RTx This is equivalent to transforming vector x back to the original coordinate system in the opposite direction.

Step 2: Multiply by ω×

According to the definition of the cross product of matrices:

ω×RTx=ω×RTx.

Therefore,

Rω×RTx=Rω×RTx.

Step 3: Use the relationship between rotation and cross-product:

By

Ra×b=Ra×Rb,

we obtain

Rω×RTx=Rω×RRTx.

Since the rotation matrix satisfies

RRT=I,

so

Rω×RRTx=Rω×x.

Furthermore, by the definition of the transpose of a matrix:

Rω×x=Rω×x.

Therefore, for any vector x, we have

Rω×RTx=Rω×x.

Therefore, the two matrices must be equal:

Rω×RT=Rω×

Intuitively, ω× represents “cross-multiplication in the direction of ω.” When the coordinate system rotates by R, the direction of the rotation axis also changes to Rω, so the corresponding new cross-product matrix is

Rω×.

II. Lemma 2

-Rω×p=p×Rω

This is because the cross product satisfies the inverse commutative law:

a×b=-b×a

Swapping the positions of the two vectors reverses the direction of the result.

Let

a=Rω,  b=p,

then

Rω×p=-p×Rω.

Multiply both sides by -1:

-Rω×p=p×Rω

A simple example:

Let

Rω=001,  p=100.

First, calculate:

Rω×p=001×100=010.

Therefore,

-Rω×p=0-10.

Calculating in reverse:

p×Rω=100×001=0-10.

The two are equal:

-Rω×p=p×Rω.

In summary:

Rω×RT=Rω×

From “Rotations and Cross-Products Are Compatible”;

-Rω×p=p×Rω

From “The Sign Changes When the Order of Transposition Is Reversed.”

SE3is defined by the following relation:

Adgξ∧=gξg-1

This means:

1.    Express the twist vector ξ as a matrix ξ;

2.    Transform the coordinate system using gξg-1;

3.    Then use the vee operator to convert the result back into a six-dimensional vector.

Therefore,

Adgξ=gξg-1∨.

This is the fundamental definition of the adjoint transformation in robotics.

Calculating gξg-1 for each twist is cumbersome, so by organizing this operation into a fixed 6×6 matrix, we obtain the quick formula.

4. The quick formula is derived from the definition

Let

g=Rp01,  g-1=RT-RTp01,

and

ξ=ω×v00.

Substituting into the definition:

gξg-1.

First multiplication:

gξ=Rp01ω×v00=Rω×Rv00.

Multiply by g-1 again:

gξg-1=Rω×RT-Rω×RTp+Rv00.

Using

Rω×RT=Rω×,

and

-Rω×p=-Rω×p=p×Rω,

we obtain

gξg-1=Rω×Rv+p×Rω00.

Take the vee:

gξg-1∨=Rv+p×RωRω.

Express the relationship on the right-hand side with respect to v and ω in matrix form:

Rv+p×RωRω=Rp×R03×3Rvω
Therefore,

Adg=Rp×R0R

Single-link check

For a link of length L rotating about the world z-axis, the end-point position and its direct time derivative are

pθ=Lcosθ,Lsinθ,0T

p=-Lsinθ,Lcosθ,0Tθ

The corresponding spatial twist has vs=0and ωs=0,0,θT. Applying the point-velocity rule gives

p=vs+ωs×p=-Lsinθ,Lcosθ,0Tθ

The result matches direct differentiation. This example also shows why a spatial twist may have zero linear component while a point away from the rotation axis still moves.

In summary, position follows θ->gθ->pθ, while velocity follows θ->g->Vs->Jsθ. The Jacobian is the bridge between joint rates and spatial motion.

From a spatial twist to point velocity

The velocity of the origin of F is obtained by applying the twist matrix to its homogeneous point coordinate:

vFs=VWFspFWvFs=JWFsΘWF∧pFW	(9)

gWF=Rp01.pFW=gWF0001.pFW=gWF0001.

Space velocity is defined as

VWFs=gWFgWF-1.

Therefore,

gWF=VWFsgWF.

Substituting, we get

pFW=VWFsgWF0001=VWFspFW.

If the space twist is written as

Vs=vω,  Vs=ωv00,

Then

Vsp1=ωp+v0=ω×p+v0.

Therefore, the true 3D point velocity is

p=ω×p+v

That is to say:

- v: the translational velocity part of the spatial twist;

- ω×p: the linear velocity brought to the point by the rigid body rotation;

- Adding the two together, we get F. The actual linear velocity of the origin in the world coordinate system.

In standard vector notation, a spatial twist [v; omega] produces point velocity v + omega x p. This operation converts the 6-by-n spatial Jacobian into the effective 3-by-n point-position Jacobian used by the controller and collision constraints.

Module subchain Jacobians

Collision constraints must be applied to every module, not only to the end effector. For module m_i, the subchain W to M_i contains only the upstream joints that can move that module.

JMis=ξ'11ξ'12⋯ξ'iji	(10)

vMis=JMisΘWMi∧pMiW	(11)

To use the same full decision vector in every constraint, the columns for downstream joints are padded with zero vectors:

JMis=ξ'11⋯ξ'iji06×1⋯06×1	(12)

vMis=JMisΘWF∧pMiW	(13)

Implementation consequence  Every module-obstacle constraint requires that module's own point Jacobian. Reusing only the end-effector Jacobian is mathematically incorrect.

Feedback Control as a Velocity Constraint

Exponential error dynamics

Let pFt be the desired task trajectory, vFs=pF its derivative, and e=pF-pF the position error e=vFs-vFs its derivative. The paper specifies the first-order stable error dynamics:

ė+Ke=0, K≻0	(14)

For constant positive-definite K, et=e-Kte0. Substitution gives:

vFs-vFs+KpF-pF=0	(15)

JWFsΘWF∧pF=vFs+KpF-pF	(16)

After converting the twist expression to a point Jacobian Jp,Fequation (16) becomes the familiar resolved-rate law Jp,FΘ=vF+KpF-pF

vFs=pFW=VWFsgWF0001=VWFspFW,

VWFs=JWFsΘWF.vFs=JWFsΘ∧pFW.
JWFsΘ∧pFW&=k=1nJksθk∧pFW&=k=1nJkspFWθk.

Arrange the velocities produced by applying a twist to each joint at the same point into a matrix:

Jp,F:=J1spFWJ2spFW⋯JnspFW1:3

Here, the subscript1:3 indicates that only the first three homogeneous components of the linear velocity are taken.

Thus, the sum above can be expressed as a matrix multiplication:

k=1nJkspFWθk=Jp,FΘ.

Therefore:

JWFsΘ∧pFW=Jp,FΘ

Multiple coupled motion goals

For alpha task frames, the individual equations are stacked:

JΘ=V+KP-P	(17)

where：

J=Jp,F1Jp,F2⋮Jp,Fα,  P=pF1⋮pFα,V=vF1⋮vFα

The global joint vector contains each physical joint once. If two task chains share a joint, the same decision variable appears in both task rows, so the coupling is handled automatically.

Minimum-norm solution and joint bounds

minΘ 12ΘTΘ

subject to JΘ=V+KP-P	(18)

Without additional inequalities and with full row rank, the minimum-norm equality solution is Θ⋆=JTJJT-1V+KP-P. The QP form allows hardware and motion constraints to be added directly.

Using one-step Euler integration,

Θ+=Θ+ΘΔt

We require

Θmin≤Θ+ΘΔt≤Θmax
, the joint-position limits become:

Θmin-ΘΔt≤Θ≤Θmax-ΘΔt	(19)

Θmin≤Θ≤Θmax	(20)

If K demands a task velocity incompatible with these bounds, the hard equality formulation can become infeasible. This motivates the later soft-task objective.

Workspace Boundary Constraints

The paper approximates each module as a sphere:

center pMi;

radius ri.

For the jth face of the environment polyhedron:

sij is the unit normal vector pointing from the module center to that face;

dij is the perpendicular distance from the module center to that face.

vMis⋅sij=JMisΘ∧pMi⋅sij≤dij	(21)

Equation (21) protects only the center. Accounting for the module sphere gives:

vMis⋅sij≤dij-ri	(22)

Dimensional caveat: Formulas (21) and (22) have different dimensions: the left side represents velocity, and the right side represents distance. The paper states that the robot maintains this velocity for a time "far less than one second."

A more rigorous discrete derivation would be:

pi+≈pi+viΔt

Therefore:

Δt viTsij≤dij-ri

That is:

viTsij≤dij-riΔt

Sphere-Tree Obstacle Approximation

The exact collision-free set Cfree is difficult to express analytically in the high-dimensional joint space. The paper therefore approximates every robot module by a sphere and every obstacle by a collection of spheres produced by a sphere-tree construction.

‖pMi-oj‖≥ri+rjo	(A)

Because pMi is nonlinear in the joint configuration, the direct sphere-sphere distance condition is nonlinear. A coarse sphere approximation is conservative and may close feasible passages; a fine approximation improves geometry but may create hundreds of constraints.

In the block example in Figure 8, the three sphere-tree levels contain 8, 64, and 470 spheres. The paper's answer is not to insert all fine-level spheres into the QP. It first constructs separating planes and uses them to reduce the obstacle set.

Separating Plane Derivation

Closed-form construction

For module sphere i and obstacle sphere j, define the unit vector from the module center to the obstacle center:

sij=oj-pi‖oj-pi‖	(B)

The point on obstacle sphere j closest to the module is:

o'j=oj-rjosij	(C)

The separating plane is tangent to the obstacle sphere at o'j:

Pij=x∣sijTx-o'j=0	(D)

The robot is on the negative side of this plane, and the obstacle is on the positive side. For the entire robot sphere to remain on the safe side, its center must satisfy sijTpi-o'j≤-ri.

Clearance and the linear velocity inequality

vi=JMisΘ∧pMi

Let dij=oj-pi. Since o'j-pi=oj-pi-rjosij=dij-rjosij, the remaining separation between the two sphere surfaces is:

hij=‖o'j-pi‖-ri=dij-rjo-ri	(E)

With pi+≈pi+viΔt, requiring the robot sphere not to cross the tangent plane gives

sijTpi+-o'j≤-ri

Substitute:

sijTpi-o'j+Δt sijTvi≤-ri

Start:

Δt sijTvi≤-ri-sijTpi-o'j=sijTo'j-pi-ri

Define:

hij=sijTo'j-pi-ri,

Rearrange:

Δt sijTvi≤hij

sijTvi≤hijΔt

The paper writes the corresponding short-step condition as:

vMis⋅sij=JMisΘ∧pMi⋅sij≤o'j-pMi-ri	(23)

At the current control cycle, p_i, s_tilde_ij, and the module Jacobian are fixed. Therefore equation (23) is affine in Θ. A dimensionally explicit implementation uses the right-hand side hijΔt.

Why is it linear with respect to QP?

Within the current control cycle:

- piis known;

- ojis known;

- sij is known;

- JMi is known.

Therefore, defining the point velocity Jacobian Jp,i, we have:

vi=Jp,iΘ

Equation (23) becomes:

sijTJp,iΘ≤hij

This is a linear inequality in the decision variable Θ.

Although both sijand Jp,ivary nonlinearly with the robot’s state, they are frozen within the current QP and recalculated in the next control cycle.

Thus, the paper does not transform the original nonlinear collision conditions into global linear constraints all at once, but rather:

constructs a new local separation plane at the current position in each control cycle to perform continuous local convexification.

Why it is fast: The plane coefficients are obtained by vector subtraction, a norm, normalization, and a radius offset. The paper does not solve a separate LP, QP, or maximum-margin problem to find the plane.

Redundant-Sphere Removal by Separating Planes

Geometric dominance test

After constructing Pij for module i and obstacle sphere j, the paper tests whether the same plane also completely separates the module from another obstacle sphere k. The robot sphere lies completely on the negative side if:

sijTpi-o'j+ri≤0	(F)

Obstacle sphere k lies completely on the positive side if:

sijTok-o'j-rko≥0	(G)

If both inequalities hold, every continuous motion from the robot side to sphere k must cross Pij. Enforcing the plane constraint for j therefore also protects against k, making k redundant for module i at the current planning step.

If sphere k intersects the plane, straddles the plane, or lies partly on the robot side, it cannot be removed by this test.

Active sphere set Si

Refine the set of obstructing balls by repeatedly applying the erase-remove idiom.

The conceptual process is as follows:

•    For module i, initialize the candidate set:
Si←S

•    For candidate ball j, construct Pij;

•    Check whether this plane completely separates the module balls from the other balls k;

•    If so, ball k is redundant for the current module;

•    Repeatedly remove balls until no more can be removed;

•    Let the remaining set be denoted as:
Si⊆S

Si is module-specific; different robot modules generally retain different spheres.

- The removal is temporary and must be recomputed as the robot moves.

- This is geometric dominance or occlusion, not merely a center-distance threshold.

- The paper gives the separation rule but does not specify a complete ordering, tie-breaking strategy, or complexity analysis for erase-remove.

Figure 9 result: A 14-module CKBot configuration is placed among six obstacles represented by 373 sphere-tree spheres. For the circled module, only five obstacle spheres and five planes remain necessary at that planning step. Obstacles 1 and 6 are hidden behind other obstacle constraints.

Integrated Control and Motion-Planning QP

Combining the hard task law, joint limits, workspace faces, and the reduced obstacle sets gives equation (24):

minΘ 12ΘTΘs.t. &JΘ=V+KP-P&Θmin-ΘΔt≤Θ≤Θmax-ΘΔt&Θmin≤Θ≤Θmax&JMisΘ∧pMi⋅sij≤dij-ri, &                           ∀Mi,fj∈VK×F&JMisΘ∧pMi⋅sik≤o'k-pMi-ri,&                           ∀Mi,Sk∈VK×Si #(24)

The objective is strictly convex, and every equality or inequality is affine in the current decision vector. The solution is therefore the minimum-norm joint velocity satisfying the current task, hardware, boundary, and reduced obstacle constraints.

Important notation: The obstacle constraints are indexed over VK×Si, not VK×S. Equation (24) therefore assumes the separating-plane reduction has already been performed.

Sequential Soft Objective

Soft task tracking: equation (25)

When a hard task equality conflicts with avoidance or hardware limits, the paper moves task tracking into the objective. Define b=V+KP-P:

Substitute the hard-constraint equation:

JΘ=b

into the objective function:

fΘ=Θ2+λJΘ-b2	(25)

This means:

•    The first term aims to keep the joint velocity low;

•    The second term aims to keep the task velocity close to the desired feedback velocity;

•    The larger λ is, the greater the emphasis on task tracking;

•    Obstacle avoidance and joint constraints remain hard constraints.

Expanded:

f=&ΘTΘ+λJΘ-bTJΘ-b=&ΘTI+λJTJΘ-2λbTJΘ+λbTb

The constant term does not affect the optimal solution.

If we use the standard form:

12ΘTHΘ+gTΘ

then we can set:

H=2I+λJTJ

g=-2λJTb

Because:

I+λJTJ≻0

The objective function remains strictly convex.

Near-obstacle normal-velocity penalty: equation (26)

When module i is closer than dmin to obstacle sphere j, the paper adds a penalty on its velocity component normal to the obstacle plane:

fΘ=Θ2+λJΘ-b2+μijvMis⋅sij2	(26)

With
aij=sijTJp,i,
the added term is
μijΘTaijTaijΘ.

It is positive semidefinite and preserves convexity.

Because:

vMis=Jp,iΘ

Definition:

aij=sijTJp,i

We obtain:

viTsij=Jp,iΘTsij.

Using the transpose of a product:

ABT=BTAT,

so

Jp,iΘT=ΘTJp,iT.

Therefore,

viTsij=ΘTJp,iTsij.

This is a scalar, so it equals its own transpose:

ΘTJp,iTsij=ΘTJp,iTsijT.

After transposing:

=sijTJp,iΘ.

Therefore,

viTsij=sijTJp,iΘ.

The penalty term is:

μijaijΘ2=μijΘTaijTaijΘ

Therefore, the Hessian increment is a positive-semidefinite matrix of rank ≤1:

Hobs∝μijaijTaij

This does not violate the convexity of QP.

Differences Between the Paper’s Description and the Mathematical Effect

The paper states that this term is used to suppress the robot’s movement “toward the obstacle.”

However, because it is a quadratic term:

viTsij2

it penalizes both:

•    the normal velocity toward the obstacle;

•    the normal velocity away from the obstacle.

In other words, it tends to bring the normal velocity close to zero, causing the module to slide along the tangent plane rather than unilaterally penalizing only approaches.

The paper removes this term upon actual contact to prevent it from hindering the robot’s ability to move away from the obstacle.

Additionally, the paper only mentions distances less than dmin, but does not explicitly specify whether it uses:

•    the center-to-center distance;

•    spherical net clearance;

•    or the distance to the separation plane.

It only states that this can be easily verified using the sphere-to-sphere distance. A faithful reproduction must explicitly select and document which measure is used.

Contact repulsion: equation (27)

If module Miencounters the jth obstacle ball, the paper requires the module to move away from the obstacle.

Because:

sij
points from the module toward the obstacle, so:

- viTsij>0: moving toward the obstacle;

- viTsij=0: motion along the tangent plane;

- viTsij<0: motion away from the obstacle.

The paper adds:

vMis⋅sij=JMisΘ∧pMi⋅sij≤γij 			(27)

Where:

-vMis≤γij≤0

If:

γij<0

then the normal velocity must be negative, forcing the robot to move away from the obstacle.

Replacing the normal vector with the one pointing from the obstacle toward the robot:

naway=-sij
the equation becomes:

nawayTJp,iΘ≥-γij
The right-hand side is non-negative, representing the minimum outward velocity.

Complete Transition During the Contact Phase

The paper explicitly states:

1. Approaching but not in contact: Add the soft normal velocity penalty from Equation (26);

2. Contact: Remove the penalty from Equation (26) that was previously added;

3. Add the hard repulsive constraint from Equation (27). In other words:

near: soft normal suppression

is switched to:

contact: hard repulsive velocity

The paper states that “the larger γij is, the faster the separation,” but according to its sign definition, a more negative γij—that is, one with a larger absolute value—would require a faster separation velocity. This is a sign ambiguity between the paper’s text and its equations.

The paper also fails to specify:

- the specific scheduling function for γij;

- contact detection tolerance;

- the hysteresis interval for the near/contact transition;

- how to allocate repulsive velocities when multiple contacts occur simultaneously.

These details must be explicitly designed by those attempting to reproduce the results; one cannot assume that the paper has already specified them.

Why the Method Is Sequential Convex Optimization

The paper does not solve the entire trajectory in a single step, but rather updates the following at each step:

•    Current Jacobian JΘ;

•    Module center pMi;

•    Boundary direction sij;

•    Obstacle direction sij;

•    separation plane Pij;

•    set of active spheres for each module Si;

•    near-obstacle penalty term;

•    repulsion constraint after contact;

•    task reference point Pt and velocity Vt.

These quantities are fixed in the current period, so it is a convex QP; they are recalculated in the next period.

Therefore:

Nonlinear robot problem

is treated as:

A series of locally linear, state-dependent convex QPs

This is closer to velocity-level sequential convex control than to the traditional SQP, which seeks a complete finite-time-domain trajectory.

Algorithm 1: Control and Motion Planning

The algorithm takes the module twist library, module graphs, base module, task frames, desired task trajectories, environment faces, and obstacle sphere set. It returns success or failure.

- Build G_K by breadth-first traversal from the base module.

- Extract the chain from W to each task frame F_i.

- Initialize the joint state Theta, gain K, time step Delta t, and time t = 0.

- Continue while the sum of task-frame errors to their final goals is at least epsilon.

- For every module-face pair, compute the current boundary normal s_hat_ij.

- For every module, compute the reduced obstacle set S_i and the current directions s_tilde_ik for retained spheres.

- Construct and solve the current QP using the objective and constraints from Sections 4.1-4.3.

- If the solver returns no valid joint velocity, terminate with failure.

- Publish Theta_dot to the robot, advance time by Delta t, and repeat.

- Terminate with success when the final task error is below epsilon.

Line 8 is essential  Algorithm 1 explicitly computes S_i subseteq S for every module before SolveQP. A reproduction that always passes every sphere to the QP has not implemented this line of the paper.

If a full desired trajectory is supplied, the method tracks it. If only a destination is supplied and the terminal time is treated as unbounded, the repeated local optimizations generate a trajectory. The output may also be integrated offline using Theta_(k+1) = Theta_k + Theta_dot_k Delta t.

Assumptions and Unspecified Details

- Local rather than globally complete planning: the method may stall in a local equilibrium or fail even when a global route exists.

- Short-step safety: equations (21)-(23) rely on a short execution interval and repeated replanning; the displayed paper equations omit an explicit Delta t.

- Conservative geometry: safety with respect to spheres implies true-geometry safety only when the sphere covers are valid conservative certificates.

- Static obstacles: the paper does not include obstacle velocity in equation (23).

- Velocity-level model: torque, inertia, Coriolis terms, acceleration bounds, and actuator dynamics are not derived.

- No systematic self-collision derivation: the theory focuses on robot-environment collision avoidance.

- Tree topology: closed kinematic loops require constraints beyond the presented graph construction.

- Conditional exponential convergence: equation (14) holds exactly only when the hard task equality is feasible and accurately executed; the soft objective trades tracking against other requirements.

- Under-specified sphere reduction: the paper states the separation test and erase-remove concept but not a deterministic ordering, tie-breaking rule, or formal complexity bound.

- Under-specified switching: d_min, gamma_ij, contact tolerance, hysteresis, and simultaneous-contact behavior require implementation choices.

Requirements for a Faithful Reproduction

- A conservative sphere model for every robot module.

- A multi-sphere or sphere-tree representation for every environment obstacle.

- Closed-form construction of o'_j, s_tilde_ij, and P_ij for each candidate module-sphere pair.

- A complete-plane separation test against other obstacle spheres.

- A deterministic erase-remove procedure producing a separate S_i for every module.

- Hard obstacle inequalities constructed only from retained sphere sets.

- Explicit far, near, and contact modes.

- Equation (26) in the near mode and removal of that penalty at contact.

- Equation (27), or a clearly documented mathematically equivalent repulsive constraint, in the contact mode.

- Reconstruction of planes, active sets, penalties, and constraints every control cycle.

- Explicit treatment of Delta t, the definition of d_min, the gamma schedule, tolerances, and hysteresis.

- Logging of raw sphere pairs, retained constraints, active penalties, contact constraints, QP setup time, and QP solve time.

Assessment of the current reproduction  The existing main reproduction includes local separating half-space constraints, a barrier-style hard condition, and part of the near-obstacle penalty. It does not yet implement the paper's separating-plane redundant-sphere removal, and it does not reproduce the equation (26) to equation (27) state switch exactly.

Method Summary

- Module connection graph

- Graph-derived forward kinematics and module Jacobians

- Module spheres and obstacle sphere trees

- Closed-form tangent separating planes

- Plane-based redundant-sphere removal

- Per-module active sets S_i

- Linear workspace and obstacle inequalities in joint-velocity space

- Sequential far/near/contact objective and constraint updates

- Convex QP solution and one-step execution

- Complete reconstruction at the next control cycle

The real-time performance therefore comes from more than the QP solver. It depends on the entire modeling chain: analytically cheap separating planes, aggressive but safe reduction of obstacle spheres, and a small convex problem that is rebuilt from the current state.

Reference

Liu, C., & Yim, M. (2021). A Quadratic Programming Approach to Manipulation in Real-Time Using Modular Robots. arXiv:2104.02755v2.

This document is an explanatory study guide. Equations retain the numbering used in the source paper; lettered equations are supplemental derivations introduced for clarity.